# REPORT TITLE # 2018

August 8 COMPANY NAME Authored by: Your Name

# CCNA

Juniper devices:

Switching Devices:

EX series: EX2200, EX2200-C, EX3300, EX4200, EX4300, EX4500, EX4600, EX6200, EX8200

Support voice, video, data

8 QoS queues /switch ports

Virtual chassis technology

QFX series:

High performance switches

Designed for enterprise, SP & DCs

Virtual chassis technology

Combine switches into on to provide one high performance switch

Routing Devices:

MX Series VMX(virtual), MXS-20, MX-10/20/2020

Edge routers designed for service providers Enterprise network

PTX series PTX 1000/3000/5000

Package transport routers, core routers designed for high volume of traffic

ACX series ACX 1000/2000/4000/5000/5048/5096

Temperature harden devices, designed for mobile backhaul, service provider network

E series E120/320/ERX1410/ERX1440

Broad band service routers, ideal for internet based services IPTV, VOIP

T series T640/1600/4000

Multi chassis core routers high powerful. Contains TX matrix can connect up to 4 T640, TX Metrix plus 4xT4000 or 8xT1600

M series M7i/10i/m120/m320

Multiservice edge routers combined capabilities of IP and MPLS

Security devices

VSRX

SRX100,110

SRX210,220,240

SRX300,550,650

SRX1400,1500

SRX3400,3600

SRX5400,5600, 5800

SRX UTM (unified threat management)

## Configuring the Hostname and password in Cisco: Cisco interface (Executive mode) en(privilege executive mode) conf t (global config mode) hostname <name of device> en conf t enable password <password that you want to keep> you can see the password by show running-config (privilege executive mode)

## To store password in encrypted format: en conf t service password-encryption if you are in global config and want to get something in privilege executive mode use :do <command> to disable encryption en conf t no service password-encryption disables future password encrypted not current on To set up the console port: en conf t line console <number on which pc is connected to> password <password that you want to keep> login end you can use cdp to detect any problem Hostname and password configuration in Juniper: To show the configuration Configureshow ConfigureEdit system -Show ¦ match host-name -Set host-name <host name that you want to keep> -Commit Set system root-authentication plain-text-password <Set password> <Set password> commit

## Cisco CDP ## EnShow cdp neighbors Serial communication has 2 port dce-data communication equipment & dte- data terminal equipment dce provide clock signal To find out type En show controllers serial 0 To set clock rate of serial communication en conf t int serial 0 clock rate <clock rate you want> to configure the ip address en conf t int serial 0 ip address <ip address> <subnet> ## To Save configuration into Cisco device: Copy running-config startup-config Enter, Enter ## To Save configuration into Juniper device: To save the current configuration without the text configuration: host1#copy running-configuration system2.cnf To save the current configuration and add the text configuration to the file in compressed format: host1#copy running-configuration system2.cnf include-text-config Save all outstanding (unsaved) configuration changes to NVS. host1#copy running-configuration startup-configuration

## Configure VLAN in Cisco Devices: After configuration of IP address in the PC/ End devices PC 1 and 3 in vlan 1 PC 2 and 4 in vlan 2 To select range of ports: interface range Fa0/4 – 8 Inside switch: en conf t int f0/2 switchport mode access switchport access vlan 1 ## Int f0/3 switchport mode access switchport access vlan 2 en conf t Int f0/1 Switchport mode trunk Reflect on all the switches

Do reflect this changes in all the switches Why we cant ping between pc 2 and pc 4 due to native vlan1- the f0/1 of both switch are assigned to native vlan only traffic from vlan 1 can pass Native vlan vlan 1 is considered as native vlan all untagged vlan packets are assume to be a native vlan packet. To enable vlan 2 we need to configure the port connecting switch with each other as trunk ports. en conf t Int f0/1 Switchport mode trunk Reflect on all the switches There are different types of encapsulation present in the switching devices. Encapsulation: ISL cisco proprietary & dot1Q – Open standards. The switches such as 3560 24 PS supports 2 types of encapsulations Therefore, to configure the trunk mode in these switches differs. You just need to configure trunk port in the following way instead of pervious to achive the result. En conf t int f0/1 Switchport trunk encapsulation dot1q Switchport mode trunk ## Configure VLAN in Juniper Devices: edit Show vlans Edit vlans Set vlan1 vlan-id 1 Set vlan2 vlan-id 2 Show Top Edit interfaces Edit ge-0/0/2 unit 0 ethernet-switching Set port-mode access Set vlan member vlan1 Up 3 Edit ge-0/0/3 unit 0 ethernet-switching Set port-mode access Set vlan member vlan1 Up 3 Edit ge-0/0/1 unit 0 ethernet-switching Set port-mode trunk Set vlan member vlan1 Set vlan member vlan2 <set all the vlans that you want the switch to flow the packets> show Up 3 Top Commit Exit Show vlans For dot1q Set interface ge-0/1/0 unit 0 family ethernet-switching port-mode trunk Set interface ge-0/1/0 unit 0 family ethernet-switching vlan members 1 2 #Set vlans v100 vlan-id 100 #Set vlans v100 l3-interface vlan.100 #Set interface ge-0/1/0 unit 100 family inet address 20.0.0.2/24 If you are configuring the cisco and juniper then use following set of commands Juniper Set interface ge-0/1/0 unit 0 family ethernet-switching port-mode trunk Set interface ge-0/1/0 unit 0 family ethernet-switching vlan members 1 2 Cisco Int f0/1 Switchport trunk allowed vlan 1,2 Switchport mode trunk Configure Cisco switch and router for inter-vlan routing: Do all configuration same as intra vlan routing at router and the switch connecting router do this: Router: Enconf tint g0/0 No shutdown Int g0/0.<vlan id 1> Encapsulation dot1q <vlan id 1> Ip address <gateway ip address of vlan id 1> Int g0/0.<vlan id 2> Encapsulation dot1q <vlan id 2> Ip address <gateway ip address of vlan id 2> Connecting Switch: Enconf tInt g0/1 Switchport mode trunk Connecting Switch: Enconf tInt g0/1 Switchport mode trunk Troubleshooting of above problem: - Confirm the problem: The 2 vlan pc could not be able to communicate in between The R1 enables the communication between the two vlans On switch Show ip interface brief Show run Show interfaces trunk -NP Check PC default gateway: Ipconfig /all-- NP On Router: Show ip interface brief Show interface g0/0.13 check vlan id = 13 – yes, NP Show interface g0/0.24 check vlan id = 14 –No, P Change vlan id to 24 Conf t int g0/0.24 Encapsulation dot1q 24 Test vlan pc with ping Configure Juniper switch and router for inter-vlan routing: Do all configuration same as intra vlan routing at router and the switch connecting router do this: Router: VSRX Set interfaces ge-0/0/1 vlan-tagging Set interfaces ge-0/0/1 unit 10 vlan-id 10 Set interfaces ge-0/0/1 unit 10 family inet address 192.168.10.1/24 Set interfaces ge-0/0/1 unit 20 vlan-id 20 Set interfaces ge-0/0/1 unit 20 family inet address 192.168.20.1/24 Commit and-quit Show configuration interfaces ¦ display set Switch side Edit interface ge-0/1/0 Set interface ge-0/1/0 unit 0 family ethernet-switching port-mode trunk Set interface ge-0/1/0 unit 0 family ethernet-switching vlan members 10 20 ###

How to set username and password to device with local device database En conf t username <name of user> password <password of user> Line console 0 Login local User name is note case sensitive Password is case sensitive To set console banner and login banner Conf t banner motd ‘message you want to set’ Banner login ‘message you want to set’

Configure basic DHCP: configure router to switch interface with ip address And loopback address router is your dhcp server and DNS server Enconf t ip dhcp excluded-address <ip from> <ip to> Ip dhcp pool <pool name> Network <Network IP to use dhcp on> <subnetmask> To know the neighbor details Sh cdp neighbours details Sh cdp entry <neighboring device name> To know device details Sh version

Default-router <default gateway IP> Dns-server <dns ip address> Do sh Ip dhcp binding to see the ipadress associated with the mac

CDP displays the directly connected neighbors to the devices Sh cdp neighbours To change the clock rate of cdp use En conf t int <interface> Clock rate <clock> To see the cdp activation Sh cdp int To enable cdp Cdp run To disable No cdp run To configure in certain interfaces Int range <startingint – Endingint > Int range f0/1 - 10 No cdp run Loop back interfaces: Loopback interfaces are logical interfaces Used for testing, never goes down Loop back interfaces are on different networks Int loopback <loopback no>

Port security: Configured on switch interfaces and controls which mac address are allowed on which interfaces. To see the mac address tables Sh mac address-table Port security is only configured on the access port not on the trunk port The port security cant take the dynamic port you have to explicitly mention the post as access port En conf t switchport mode access Interface range f0/2 - 3 switch port-security switchport port-security mac-address <mac address> or switchport port-security mac sticky switchport port-security violation shutdown to see port security do show port security what if the port security violated 3 action it can take - Protect : drop packet for mac address that are not allowed on the port. No shut down of interface , doesn’t display any message don’t increase the violation count the swich keeps. - Restrict. Dorp packet, display message on switch and increase the violation count on the switch. - Shutdown. Put interface into shutdown error disabled state increase violation count Default is shutdown To set it use : switchport port-security violation shutdown to see the mac address of secure port sh port-security address Configure VTY line for Telenet and SSH: VTY line Telenet: Virtual teletype The devices are connected to Management PC via regular ethernet port. It use vty lines in network devices for connection. On switch: En conf t int vlan 1 ip address <management ip you want to put > <subnet> no shut On router: En conf t int <interface connecting device> ip address <management ip you want to put > <subnet> no shut On all devices: Username <user> password <pass> Line vty 0 15 Login local Transport input telenet Pc telenet <management ip of device > Configure VTY line for SSH: VTY line ssh: secure shell The devices are connected to Management PC via regular ethernet port. It use vty lines in network devices for connection. On switch: En conf t int vlan 1 ip address <management ip you want to put > <subnet> no shut On router: En conf t int <interface connecting device> ip address <management ip you want to put > <subnet> no shut On all devices: Username <user> password <pass> Ip domain-name <domainname.com> Crypto key generate rsa 1024 <select size of key> Line vty 0 15 Login local Transport input ssh Exec-timeout <select inactivity time to logout> Exit Ip ssh version 2 Don’t forget to put the ip of the management pc in same subnet Pc ssh -l <username> <management ip of device > Routing: Static routing: Once Ips are configured En conf t ip route <ip of the network not connected directly to the router> <its subnet> <ip of opposite routers interface ip address/ or you can give exit interface > To see the routing table Sh ip route C means connected network L refers as local address of router S for staic route Configure same on opposite router To set floating backup static route: En conf t ip route <ip of the network not connected directly to the router> <its subnet> <ip of opposite routers interface ip address/ or you can give exit interface > <AD 1 higher than the current dynamic routing AD> Dynamic routing protocol: RIP: Router rip Network <interfaces of network connecting other routers> Network <router connecting lower networks> ACL: Standard ACL controls the traffic based on source address where as the extended ACL controls the traffic based on source and destination address. Number or name and standard or extended. 1-99 for standard access list Number standard access list: ACL must be configured closed to the destination Access-list <number> <action> <network> <network wildcard> Access-list 1 permit 192.168.1.0 0.0.0.255 This mean only allow 192.168.1.0 traffic and deny all other Everything doesn’t apply will be denied After configuring the access list you need to apply it. Applying ACL to interface: Get into interface Int g0/1 Ip access-group 1 out Ip access-group <access-list number> <in/out> To deny certain host: Access-list 1 deny host 192.168.2.4 Access-list 1 permit any Int g0/2 Ip access-group 1 out The access list check hierarchical conditions if 1 one doesn’t satisfied then. It will go for next condition. Extended ACL: To allow certain host to access certain host: Access-list <100-199> permit ip host <source host ip> host <destination host ip> Access-list <100-199> deny ip any host <destination host ip> R1(config)#access-list 100 permit ip host 192.168.1.3 host 192.168.3.3 R1(config)#access-list 100 deny ip any host 192.168.3.3 To allow certain network to access certain hosts: Access-list <100-199> permit ip host <source network ip> <network wild card> host <destination host ip> Access-list <100-199> deny ip any host <destination host ip> R1(config)#access-list 100 permit ip 192.168.2.0 0.0.0.255 host 192.168.3.4 R1(config)#access-list 100 deny ip any host 192.168.3.4 Applying to the interface: R1(config)#int gigabitEthernet 0/0 R1(config-if)#ip access-group 100 out To configure named access lists: Ip access list standard <name of accesslist> Deny 192.168.2.0 0.0.0.255 Permit any Exit Interface <interface where you want to block traffic form ip> Ip access-group <accesslist name> out NTP: Cisco devices has hardware calender built in to look Sh clock Sh clock detail Configuration of timing done in global config mode Clock timezone <timezone shortform> <hrs offset> <min offset> Come back in privilege mode and configure the clock Clock set <hh mm ss> <Month date year> Sh clock detail To set is as NTP configure it as master to configure and syc with all devices for client configure it as server Master device: Ntp master <authoritative you want lower is more autority> Client devices Global config mode Clock timezone <timezone shortform> <hrs offset> <min offset> Ntp server <ip address of link where ntp master is set> To check NTP Sh ntp associates sh ntp associations for indirect devices: Clock timezone <timezone shortform> <hrs offset> <min offset> Ntp server <ip address of link where ntp intermediate device is set> Setting up NTP with authentication: clock set 08:43:00 Apr 06 2020 Conf t clock timezone MDT -4 ntp authenticate ntp authentication-key 1 md5 cisco1 ntp authentication-key 2 md5 cisco2 R1(config)#ntp trusted-key 1 R1(config)#ntp trusted-key 2 Other devices: Clock timezone EDT -4 Ntp authenticate Ntp authentication-key 1 md5 cisco1 Ntp trusted-key 1 Ntp server 12.0.0.1 key 1 Sh ntp associations LLDP (Link layer discovery protocol) Conf t no cdp run lldp run (on all the devices) Sh lldp Sh lldp neighbors sh lldp neighbors detail to remove lldp in the interface: no lldp receive no lldp transmit Setting UP DHCP:# to do this you need to config router with their interface address and routing protocol: Enconf t ip dhcp pool <name of pool> network <network address of pool> <subnet> network 10.0.0.0 255.255.255.0 default-router <default router ip> dns-server <DNS server ip> ip dhcp excluded-address <ip of excluded address> <ip of excluded address> other routers config in same router: ip dhcp pool <name of pool> network <network address of pool> <subnet> network 20.0.0.0 255.255.255.0 default-router <default router ip> dns-server <DNS server ip> ip dhcp excluded-address <ip of excluded address> <ip of excluded address> Link between the routers: ip dhcp pool <name of pool> network <network address of pool> <subnet> network 192.168.12.0 255.255.255.0 int ge <interface to other router> ip address <ip of connecting interface> ip address 192.168.12.1 255.255.255.0 In other router on the connecting interface: # to get ip to connecting interface on other router R2(config)#int gigabitEthernet 0/0 R2(config-if)#ip address dhcp Interaface ge <interface connecting subnet where dhcp needs to be relayed> Ip helper-address <ip of dhcp router interface connecting relay router> Int ge 0/1 ip helper-address 192.168.12.1 DNS server: Router_1(config)#ip dhcp excluded-address 192.168.1.1 Router_1(config)#ip dhcp excluded-address 192.168.1.1 192.168.1.10 Router_1(config)#ip dhc Router_1(config)#ip dhcp p Router_1(config)#ip dhcp pool Router_1(config)#ip dhcp pool pool Router_1(config)#ip dhcp pool pool1 Router_1(config)#ip dhcp pool pool1 Router_1(dhcp-config)#de Router_1(dhcp-config)#default-router 192.168.1.1 Router_1(dhcp-config)#dh Router_1(dhcp-config)#dn Router_1(dhcp-config)#dns-server 20.0.0.100 Go in service in select DNS add the A record with name and ip address and you are good to go. For other devices where ip is manually configured can see the default gateway and DNS Ip default-gatway <default gateway of network > Ip name-server <dns ip address> STATIC NAT: Interface <inside interface of inside network> Ip nat inside Interface <outside interface connecting internet> Ip nat outside Router-1(config)#ip nat inside source static 192.168.1.11 1.2.3.11 ip nat inside source static <inside host ip address translated ip address> Router-1(config)#ip nat inside source static 192.168.1.12 1.2.3.12 ip nat inside source static <inside host ip address translated ip address> show ip nat translation show ip nat statistics show ip route Dynamic ACL: Interface <inside interface of inside network> Ip nat inside Interface <outside interface connecting internet> Ip nat outside access-list 1 permit <inside network ip address > <widecard bit> Router-1(config)#access-list 1 permit 192.168.1.0 0.0.0.255 ip nat pool <pool name > <starting ip of the range of global ip used in nat> <ending ip of the range of global ip used in nat> <netmask> Router-1(config)#ip nat pool pool1 1.2.3.10 1.2.3.20 netmask 255.255.255.0 ip nat inside source list <access list number> pool <pool name> Router-1(config)#ip nat inside source list 1 pool pool1 Sh ip nat translations Clear ip nat translations ¦¦¦Static NAT 1 to 1 translation Dynamic NAT Many to Many translations PAT is Many to 1 traslation. Uses layer 4 port number¦¦¦ PAT: Interface <inside interface of inside network> Ip nat inside Interface <outside interface connecting internet> Ip nat outside access-list 1 permit <inside network ip address > <widecard bit> Router-1(config)#access-list 1 permit 192.168.1.0 0.0.0.255 ip nat inside source list 1 interface <outside interface of the device where the nat will be used> overload ip nat inside source list 1 interface s0/3/0 overload Syslog: Indicate status of device including port down or problem. Configure device to display the message real time on cli Stored in ram or server. There are 8 levels of severity 0 = emergency, 7 = debug As you restart the link you will get: %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to down Router-1(config-if)#no shutdown Router-1(config-if)# %LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up Now these message on the cli show the level 5 which is informational To get the logs with timestamps service timestamps log datetime msec syslog messages are not displayed default on vty lines. To enable: Router-1#terminal monitor Syncronus login – syslog appeat in typing ios rewrite the command Line con 0 Logging synchronous Line vty 0 15 Logging synchronous To store the logs in ram To see the logging Sh logging To enable Router-1(config)#logging buffered To increase buffer size Router-1(config)#logging buffered 8192 To get the log in server: Logging <ip address of server> Logging host <ip address of server> Router-1(config)#logging host 192.168.1.4 See in services in syslog Password recovery, configuration backup, IOS upgrade: To recover the forgotten password from the device: Restart device physically and press control + break rommon 1 > confreg 0x2142 rommon 2 > reset en Router#copy startup-config running-config Reset password Enable secret <new pass> Router(config)#config-register 0x2102 TFTP back up Router-1#copy startup-config tftp Address or name of remote host []? 192.168.1.3 Upgrading the IOS image: To check version: Sh version Router-1#copy tftp flash Address or name of remote host []? 192.168.1.3 Source filename []? c2900-universalk9-mz.SPA.155-3.M4a.bin Destination filename [c2900-universalk9-mz.SPA.155-3.M4a.bin]? 33591768 bytes copied in 1.816 secs (1942178 bytes/sec) Router-1#sh flash: System flash directory: File Length Name/status 3 33591768 c2900-universalk9-mz.SPA.151-4.M4.bin 4 33591768 c2900-universalk9-mz.SPA.155-3.M4a.bin [67439355 bytes used, 188304645 available, 255744000 total] 249856K bytes of processor board System flash (Read/Write) Router-1#delete flash:c2900-universalk9-mz.SPA.151-4.M4.bin Delete filename [c2900-universalk9-mz.SPA.151-4.M4.bin]? Delete flash:/c2900-universalk9-mz.SPA.151-4.M4.bin? [confirm] Router-1#sh flash IPv6 Configuration: Router-0(config)#ipv6 unicast-routing Router-0(config)#interface gigabitEthernet 0/0 Router-0(config-if)#ipv6 address 2001:db8:123:123::1/64 Router-0(config)#interface gigabitEthernet 0/1 Router-0(config-if)#ipv6 address 2001:db8:1:1::1/64 Router-0(config)#do sh ipv6 int br There are address that are configured and other addresses are link local addresses automatically generated on ipv6 interfaces. These addresses are never routed used in local link. In fe80/10 range. On other routers: Configure the inner links connecting end devices. The router links will be configured using slack. Router-1(config)#ipv6 unicast-routing Router-1(config)#int gigabitEthernet 0/1 Router-1(config-if)#ipv6 add Router-1(config-if)#ipv6 address 2001:db8:2:2:::1/64 % Incomplete command. Router-1(config-if)#ipv6 address 2001:db8:2:2::1/64 Router-1(config-if)#no shut Router-1(config-if)#no shutdown Stateless address auto config It gives ipv6 address to router connecting other routers It uses ndp It will take the prefix and uses its own mac to generate address. Router-2(config)#interface gigabitEthernet 0/0 Router-2(config-if)#no sh Router-2(config-if)#no shutdown Router-2(config-if)# %LINK-5-CHANGED: Interface GigabitEthernet0/0, changed state to up %LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0, changed state to up Router-2(config-if)# Router-2(config-if)#ipv6 add Router-2(config-if)#ipv6 address au Router-2(config-if)#ipv6 address autoconfig Static router config: Router-0(config)#ipv6 route 2001:DB8:3:3::1/64 2001:DB8:123:123:201:63FF:FE26:A001 Router-0(config)#ipv6 rout Router-0(config)#ipv6 route 2001:DB8:2:2::1/64 2001:DB8:123:123:201:42FF:FE9E:4301 Router-1(config)#ipv6 route 2001:DB8:1:1::1/64 2001:DB8:123:123::1 Router-1(config)# Router-1(config)#ipv6 route 2001:DB8:3:3::1/64 2001:DB8:123:123:201:63FF:FE26:A001 If any routing issue Check the ip protocol Sh ip protocols If passive interface meaning the interface can listen to the advertisements but cant respond Router rip No passive-interface <interface that is passive> Sh ip rip database To check dhcp Sh ip interface Check for helper address Ip helper-address <interface ip is coming from> To check nat Do sh ip nat translations Do sh ip nat stat Do sh run¦ inc ins Do sh access-list To check dns In dhcp server Do sh run To check DTP: do sh in g0/1 sw Switch-1(config)#interface gigabitEthernet 0/1 On each interface disable DTP: Switch-1(config-if)#switchport nonegotiate Switch-1(config-if)#switchport mode trunk Switch-1(config)#interface range f0/1 - 10 Switch-1(config-if-range)#switchport mode access Switch-1(config-if-range)#switchport no Switch-1(config-if-range)#switchport nonegotiate Do in to all the switches…. VTP config: On each interface disable DTP: Switch-1(config-if)#switchport nonegotiate Switch-1(config-if)#switchport mode trunk VTP config: Switch-2(config)#vtp mode transparent Switch-2(config)#vtp domain cisco Switch-2(config)#vtp version 2 Switch-2(config)#vlan 40 Switch-2(config-vlan)#name accounting ON switch 1: Switch-1(config)#vtp domain cisco Switch-1(config)#vtp version 2 Switch-1(config)#vtp mode server Switch-1(config)#vlan 10 Switch-1(config-vlan)#name HR Switch-1(config)#vlan 20 Switch-1(config-vlan)#name sales Switch-1(config)#vlan 30 Switch-1(config-vlan)#name engineering On swich 3: Switch-3(config)#int range fastEthernet 0/2-3 Switch-3(config-if-range)#switchport mode access Switch-3(config-if-range)#switchport access vlan 30 Switch-3(config)#int fastEthernet 0/1 Switch-3(config-if)#switchport mode access Switch-3(config-if)#switchport access vlan 10 Switch-3(config-if)#exit Switch-3(config)#int fastEthernet 0/4 Switch-3(config-if)#switchport mode access Switch-3(config-if)#switchport access vlan 20 Switch-3(config-if)#switchport nonegotiate Switch-2(config)#int range f0/1 -2 Switch-2(config-if-range)#switchport mode access Switch-2(config-if-range)#switchport access vlan 40 Switch-2(config-if-range)#switchport nonegotiate Switch-2(config-if-range)#exit Switch-1(config)#int range fa 0/1-2 Switch-1(config-if-range)#switchport mode access Switch-1(config-if-range)#switchport access vlan 10 Switch-1(config-if-range)#switchport nonegotiate Switch-1(config-if-range)#exit Switch-1(config)#int fa 0/3 Switch-1(config-if)#switchport mode access Switch-1(config-if)#switchport access vlan 20 Switch-1(config-if)#switchport nonegotiate Switch-1(config-if)#exit On the trunk port: Switch-1(config)#int gigabitEthernet 0/2 Switch-1(config-if)#switchport trunk allowed vlan 1,10,20 TO verify: Switch-1(config-if)#do sh int trunk On switch 2: Switch-2(config)#int range gigabitEthernet 0/1 - 2 Switch-2(config-if-range)#switchport trunk allowed vlan 1,10,20 On switch 3: Switch-3(config)#int gigabitEthernet 0/1 Switch-3(config-if)#switchport trunk allowed vlan 1,10,20 Do sh vtp status Do sh vlan br Sh spanning-tree Ether channel: Layer 2 ether channel switch1 and switch 2: PAGP Switch1: Requirements: All ports must have same config duplex, speed native vlan and allowed vlan and same trunk and access ports Enconft Channel-group 1 mode desirable Do show eth sum Int po1 Switchport mode trunk Switch 2: Enconft Channel-group 1 mode desirable Do show eth sum Int po1 Switchport trunk encapsulation dot1q Switchport mode trunk Layer 3 ether channel: Ip routing Int range g0/1-2 No switchport channel-group 1 mode on int po1 ip address 23.0.0.1 255.255.255.0 Switch 3 Ip routing Int range g0/1-2 No switchport channel-group 1 mode on int po1 ip address 23.0.0.1 255.255.255.0 Now use LACP: int range fastEthernet 0/1-4 channel-group 2 mode active switchport trunk encapsulation dot1q switchport mode trunk Switch 4: int range fastEthernet 0/1-4 channel-group 2 mode active switchport trunk encapsulation dot1q switchport mode trunk Troubleshooting: Sh eth sum Sh int f0/4 To change speed Int f0/4 Speed 100 Sh Ip int g0/2 Inter-VLAN routing: Using SVI To make switch as router Enconf t Ip routing Int g0/1 Switchport access vlan 10 Int g0/2 Switchport access vlan 20 Int vlan 10 Ip address 10.0.1.1 255.255.255.0 No shutdown Int vlan 20 Ip address 10.0.2.1 255.255.255.0 No shutdown Do sh vlan br OSPF: Single area OSPF Router-1: Router ospf <pid> All the instance you will use single ospf instance on a router . You can create sepeater pid which will create separate database. The pid is locally significant they can be different in router and still they can be neighbors. Router ospf 1 Advertise network It will look for the interfaces within the range stated in the network command and advertises the network prefix on those interfaces. Doesn’t necessarily advertise the exact network in network command Network <ip of network> <wild card mask> area <area no> Even if you enter incorrect wild card the network takes correct subnet mask bits. Router-1(config-router)#network 12.0.0.0 0.0.0.255 area 0 Router-1(config-router)#network 12.0.0.0 0.0.0.255 area 0 Router-1(config-router)#network 14.0.0.0 0.0.0.255 area 0 Router-1(config-router)#network 1.1.1.1 0.0.0.0 area 0 Router2: Router ospf 2 To configure network into all interfaces Network 0.0.0.0 255.255.255.255 area 0 Router-1(config)#router ospf 2 Router-1(config-router)#network 12.0.0.0 0.0.0.255 area 0 Router-1(config-router)#network 23.0.0.0 0.0.0.255 area 0 Router-1(config-router)#network 2.2.2.2 0.0.0.0 area 0 Router-3: Router ospf 3 Network 0.0.0.0 255.255.255.255 area 0 Router-4: Router ospf 4 Network 0.0.0.0 255.255.255.255 area 0 Don’t use this short cut to realtime router as it applies ospf to all the ports. To check the neighbor relation Do sh ip ospf neighbor Do sh ip route To disable the advertisement on loopback Passive-interface l0 To calculate cost the the bandwith is devided by the reference bandwidth The reference bandwidth by default is 100MBPS fast ethernet bandwidth is 100MBPS. Therefore the metric is 1 for fast ethernet The lowest possible metric is 1. For Gigabit 1000 MBPS 100M/1000M= 0.1 Therefore GE and FA has same metric by default 10GE and 100GE is same To change reference bandwidth Auto-cost reference-bandwidth 10000 To raise the cost of certain route Int g0/0 Ip ospf cost 10000 OSPF multi area: Router0: Router opsf 1 Network 0.0.0.0 255.255.255.255 area 1 Passive-interface l0 Auto-cost reference-bandwidth 10000 Router1: Router opsf 1 Network 0.0.0.0 255.255.255.255 area 1 Passive-interface l0 Auto-cost reference-bandwidth 10000 Router2: Router opsf 1 Network 0.0.0.0 255.255.255.255 area 1 Passive-interface l0 Auto-cost reference-bandwidth 10000 Router4: Router opsf 1 Network 0.0.0.0 255.255.255.255 area 0 Passive-interface l0 Auto-cost reference-bandwidth 10000 Router7: Router opsf 1 Network 0.0.0.0 255.255.255.255 area 2 Passive-interface l0 Auto-cost reference-bandwidth 10000 Router6: Router opsf 1 Network 0.0.0.0 255.255.255.255 area 2 Passive-interface l0 Auto-cost reference-bandwidth 10000 Router8: Router opsf 1 Network 0.0.0.0 255.255.255.255 area 2 Passive-interface l0 Auto-cost reference-bandwidth 10000 Router3: Router opsf 1 Network 12.1.0.2 0.0.0.255 area 1 Network 12.2.1.1 0.0.0.255 area 0 Network 4.4.4.4 0.0.0.255 area 0 Passive-interface l0 Auto-cost reference-bandwidth 10000 Router(config-router)#area 0 range 12.0.0.0 255.0.0.0 Router5: Router opsf 1 Network 12.2.2.2 0.0.0.255 area 0 Network 6.6.6.6 0.0.0.255 area 0 Network 12.3.0.1 0.0.0.255 area 2 Passive-interface l0 Auto-cost reference-bandwidth 10000 Router(config-router)#area 0 range 12.0.0.0 255.0.0.0 OSPF troubleshooting Sh ip ospf neigh Sh ip protocols Router1: Router ospf 1 Network 1.1.1.1 0.0.0.0 area 1 Passive interface l0 Router4: Sh ip ospf neigh Router ospf 1 No passive interface g0/0 Router 2: Sh ip ospf neigh Sh ip protocol Sh ip interface br Sh run Conf t Int f0/2 No shutdown Router 3: Sh ip ospf neigh Sh ip protocol Router ospf 1 Network 10.23.0.0 0.0.0.255 area 0 No area 2 range 10.0.0.0 255.0.0.0 Area 0 range 10.0.0.0 255.0.0.0 Do sh run Router ospf 1 Area 0 range Router5: Sh ip ospf neigh Sh ip route IPV6 ospf mutiarea setup: Internet: Router(config-if)#ipv6 address 2001:DB8:1:1::1/64 Router(config-if)#no shut Router 0: Router(config)#ipv6 unicast-routing Router(config)#ipv6 router ospf 1 Now ospf process is created To activate on int g0/0 and g0/1 Router 0: Router(config-rtr)#int g0/0 Router(config-if)#ipv6 ospf 1 area 1 Router(config-if)#int g0/1 Router(config-if)#ipv6 ospf 1 area 1 Router 2 Router(config-if)#ipv6 unicast-routing Router(config)#interface gigabitEthernet 0/1 Router(config-if)#ipv6 ospf 1 area 1 Router 1: Router(config)#ipv6 unicast-routing Router(config)#interface gigabitEthernet 0/1 Router(config)#interface gigabitEthernet 0/1 Router(config-if)#ipv6 ospf 1 area 0 Router(config-if)#interface gigabitEthernet 0/0 Router(config-if)#ipv6 ospf 1 area 1 Router 3: Router(config)#ipv6 unicast-routing Router(config)#interface gigabitEthernet 0/1 Router(config-if)#ipv6 ospf 1 area 0 Router(config-if)#interface gigabitEthernet 0/0 Router(config-if)#ipv6 ospf 1 area 2 Router 4: Router(config)#ipv6 unicast-routing Router(config-if)#interface gigabitEthernet 0/0 Router(config-if)#ipv6 ospf 1 area 2 Sh ipv6 protocol Sh ipv6 ospf Sh ipv6 ospf neigh The Router 0 is connected to internet in order to make it as default route and advertise. Ipv6 routes # ip address of opposite side router internet. Router(config)#ipv6 route ::/0 2001:DB8:01:01::1 Router(config)#ipv6 router ospf 1 Router(config-rtr)#default-information originate Sh ipv6 routes EIGRP: Enhanced version of IGRP. Form cisco proprietary now moved to open standard However in multi vendor environment mostly ospf is used. Router eigrp <Autonomous system number> Router eigrp 100 The eigrp number has to be same for the all router If AS number is not same the router will ne become neighbor with other router Troubleshooting look for AS number Network <ip address> <wild card mask> Router 0 Router(config)#router eigrp 100 Router(config-router)#networ Router(config-router)#network 10.12.0.0 0.0.0.255 Router(config-router)#network 10.23.0.0 0.0.0.255 Router(config-router)#network 1.1.1.1 0.0.0. Router 1 Router eigrp 100 Router(config-router)#network 10.12.0.0 0.0.0.255 Router(config-router)#network 10.14.0.0 0.0.0.255 Router(config-router)#network 2.2.2.2 0.0.0.0 Router(config-router)#passive-interface l0 Router(config-router)#no auto-summary Router 2 Router(config)#router eigrp 100 Router(config-router)#network 10.23.0.0 0.0.0.255 Router(config-router)#network 10.34.0.0 0.0.0.255 Router(config-router)#network 10.35.0.0 0.0.0.255 Router(config-router)#network 3.3.3.3 Router 3 Router(config)#router eigrp 100 Router(config-router)#network 10.14.0.0 0.0.0.255 Router(config-router)#network 10.34.0.0 0.0.0.255 Router(config-router)#network 4.4.4.4 0.0.0.0 Router(config-router)#passive-interface l0 Router(config-router)#no auto-summary Router 4 Router(config)#router eigrp 100 Router(config-router)#network 10.35.0.0 0.0.0.255 Router(config-router)#network 5.5.5.5 0.0.0.0 Router(config-router)#passive-interface l0 Router(config-router)#no auto-summary Sh ip route OSPF can do same cost load balancing Where as eigrp can do unequal cost load balancing To enable router to load balance between links If enter 2 the load balance happen upto double the fisable distance With out fisable successor eigrp can do load balancing Router(config-router)#variance 2 Router(config-router)# %DUAL-5-NBRCHANGE: IP-EIGRP 100: Neighbor 10.12.0.1 (GigabitEthernet0/1) is up: new adjacency %DUAL-5-NBRCHANGE: IP-EIGRP 100: Neighbor 10.14.0.1 (GigabitEthernet0/0) is up: new adjacency Manual summary: To configure summary route Interface G0/0 Router(config-if)#ip summary-address eigrp 100 10.0.0.0 255.0.0.0 Eigrp troubleshooting: Sh ip eigrp neighbours Process no is 10 This process no is AS It should use AS no 100 Sh run Reconfigure eigrp with AS no 100 Sh ip protocols Sh ip int br Reconfigure loopback interface Remove passive interface of physical link Remove 10 network Eigrp network command is default to /8 network Sh run Other interface for sum Reconfigure the summary route IPV6 EIGRP: Router 1: Router(config)#ipv6 unicast-routing Router(config)#ipv6 router eigrp 100 Router(config-rtr)#passive-interface g0/2 Router(config-rtr)#int g0/0 Router(config-if)#ipv6 eigrp 100 Router(config-if)#int g0/1 Router(config-if)#ipv6 eigrp 100 Router(config-if)#int g0/2 Router(config-if)#ipv6 eigrp 100 Router(config-if)#ipv6 router eigrp 100 Router(config-rtr)#no shutdown Router 2: Router(config)#ipv6 unicast-routing Router(config)#interface g0/0 Router(config-if)#ipv6 eigrp 100 Router(config-if)#interface g0/1 Router(config-if)#ipv6 eigrp 100 Router(config-if)#ipv6 router eigrp 100 Router(config-rtr)#no shutdown Router 3: Router(config)#ipv6 unicast-routing Router(config)#interface gigabitEthernet 0/0 Router(config-if)#ipv6 eigrp 100 Router(config-if)#interface gigabitEthernet 0/1 Router(config-if)#ipv6 eigrp 100 Router(config-rtr)#no shutdown Router 4: Router(config)#ipv6 unicast-routing Router(config)#interface gigabitEthernet 0/0 Router(config-if)#ipv6 eigrp 100 Router(config-if)#interface gigabitEthernet 0/1 Router(config-if)#ipv6 eigrp 100 Router(config-rtr)#no shutdown Router 5: Router(config)#ipv6 unicast-routing Router(config)#interface gigabitEthernet 0/0 Router(config-if)#ipv6 eigrp 100 Router(config-if)#interface gigabitEthernet 0/1 Router(config-if)#ipv6 eigrp 100 Router(config-rtr)#no shutdown TO set default routes: Int g0/0 Ipv6 summary-address eigrp 100 ::/0 Int g0/1 Ipv6 summary-address eigrp 100 ::/0 PPP: Point to point protocol Layer 2 protocol used over serial WAN On cisco router the default layer 2 encapsulation is cisco hdlc PPP provides secure security through autnetication functionality. PPP with 2 different authentication Hdlc static password for authentication Chap chllenge handshake authentication protocol Setting up VPC: Nexus 9000 switches Configuration is a four step process - Enable features - Keep-Alive link - vPC Domain - Peer-Link - Member ports - Orphan ports Switch_1# conf t feature vpc feature lacp interface mgmt 0 Ip address <ip address of management interface/vlan subnet> Ip address 10.244.102.1/30 No shut Ping 10.244.102.2/30 vrf management Exit Vpc domain 10 Role priority 10 Peer-keepalive destination 10.244.102.2 source 10.244.102.1 vrf management Show vpc brief Interface eth 1/53-54 Description **vPC Peer-Link** Channel-group 15 mode active No shut Int port-channel 15 Description **vPC Peer-Link** No shut Switchport Switchport mode trunk Int eth 1/20 Channel-group 20 mode active No shut Interface port-channel 20 Switchport Vpc 20 Sh vpc brief Interface eth 1/11 No shut Switch Show vpc orphan-ports Switch_2# conf t feature vpc feature lacp interface mgmt 0 Ip address <ip address of management interface/vlan subnet> Ip address 10.244.102.2/30 No shut Ping 10.244.102.1/30 vrf management Exit Vpc domain 10 Role priority 20 Peer-keepalive destination 10.244.102.1 source 10.244.102.2 vrf management Interface eth 1/53-54 Description **vPC Peer-Link** Channel-group 15 mode active No shut Int port-channel 15 Description **vPC Peer-Link** No shut Switchport Switchport mode trunk Sh vpc brief Routing over VPC s Setting up Basic BGP internal and external: En conf t no ip domain-lookup Hostname <router name > Alias exec s show ip int br Line con 0 No exec-timeout Logging synchronous Int <interface on which you want to configure your bgp> Ip address <ip address you want to provide for the interface> <subnet mask> No shutdown Configure it ip address on both side such that you can ping using both the routers En conf t Router bgp <own AS number of router> Neighbor <ip address of other routers interface> remote-as <for i bgp own AS no for ebgp other routers AS> Same config on other router

Follow above procedure on all the routers Now the OSPF is configured & you can able to ping the neighboring routes Configuring MPLS: This config assign mpls label to packet dynamically to ip address. En conf t Mpls ip Mpls label range 100 150 Interface g<interface> Mpls ip For static binding: En conf t Mpls label range <> static <range of label for static label assignment> Mpls static binding ipv4 <ip address > <subnet> <label to bind from static range> Debug : Show mpls label range Show mpls static binding ipv4 Show mpls forwarding table Sh mpls static crossconnect At PE configure static at P configure dynamic

PPP layer 2 protocol often used over serial WAN. On CISCO router the default layer 2 encapsulation is CISCO HDLC. However, PPP provides secure security through authentication functionality. PAP static password to authenticate the connection. CHAP challenge handshake authentication protocol. Which uses the dynamic randomly generated string. Provide greater security than PAP

R1 – R1se PAP

R1(config)#username Packet password Tracer

R1(config)#username cisco password ccna

R1#sh int S2/0

R1(config-if)#ppp authentication pap

R1(config-if)#ppp pap sent-username cisco password ccna

R1se:

R1SE(config)#username cisco password ccna

R1SE(config)#int serial 2/0

R1SE(config-if)#encapsulation ppp

R1SE(config-if)#ppp authentication pap

R1SE(config-if)#ppp pap sent-username cisco password ccna

R1SE(config-if)#do sh ip int br

R2 – R2se CHAP

Username of other router.

R2(config)#username R2SE password CCNA

R2(config)#int serial 2/0

R2(config-if)#encapsulation ppp

R2(config-if)#ppp authentication chap

R2(config-if)#no shutdown

R2SE:

Router(config)#hostname R2SE pass

R2SE(config)#username R2 password CCNA

R2SE(config)#int serial 2/0

R2SE(config-if)#encapsulation ppp

R2SE(config-if)#ppp authentication chap

R2SE(config-if)#no shut

Sh int s0/0

Sh run

Password not set

Ppp pap sent-username cisco password CCNA

Authentication check

Do sh ip int br

Check hostname in chap as we use username of other router

Multilink PPP:

PAP on R1 to spr1

Chap on R2 to SPR 2

Conf t

Username Packet passward Tracer

Interface multilink 1

Ppp multilink

Ppp multilink group 1

Ppp authentication pap

Ppp pap sent-username cisco password CCNA

Ip address 100.0.0.2 255.255.255.254

Do sh run int multi 1

Int s1/0

Encapsulation ppp

Ppp multilink group 1

No shut

Int s1/1

Encapsulation ppp

Ppp multilink group 1

Show ppp all

Show ppp multilink

Ip route 0.0.0.0 0.0.0.0 100.0.0.1

Do sh ip route

R2:

Conf t

Username SPR2 password CCNA

Interface multilink 1

Ppp multilink

Ppp multilink group 1

Ppp authentication chap

Ip address 200.0.0.2 255.255.255.254

Do sh run int multi 1

Int s1/0

Encapsulation ppp

Ppp multilink group 1

No shut

Int s1/1

Encapsulation ppp

Ppp multilink group 1

Show ppp all

Show ppp multilink

Configure static default routes no router

Ip route 0.0.0.0 0.0.0.0 200.0.0.1

Do s hip route

PPPOE:

PPP authentication over ethernet frames.

Conf t

Username packet password Tracer

Interface dialer 1

Mtu 1492

Encapsulation ppp

Ip address negotiated

Ppp authentication pap

Ppp pap sent-username Cisco password CCNA

Dialer pool 1

Do sh run int dialer 1

Int g0/0

Pppoe client dail-pool-number 1

No shut

Sh ip int br

Dialer provides layer 3 information and virtual access provides layer 2 info

Ip route 0.0.0.0 0.0.0.0 100.0.0.1

Conf t

Username SPR2 password CCNA

Interface dialer 1

Mtu 1492

Encapsulation ppp

Ip address negotiated

Ppp authentication chap

Dialer pool 1

Do sh run int dialer 1

Int g0/0

Pppoe client dail-pool-number 1

No shut

Sh ip int br

Dialer provides layer 3 information and virtual access provides layer 2 info

Sh int dailer 1

Ip route 0.0.0.0 0.0.0.0 200.0.0.1

GRE tunnel:

R1(config)#int t0

R1(config-if)#ip add 192.168.1.1 255.255.255.252

R1(config-if)#tunnel source serial 0

R1(config-if)#tunnel destination 200.0.0.2

R1(config-if)#exit

R1(config)#ip route 0.0.0.0 0.0.0.0 100.0.0.1

Router(config)#hostname R2

R2(config-if)#ip address 192.168.1.2 255.255.255.252

R2(config-if)#tunnel source serial 0

R2(config-if)#tunnel destination 100.0.0.2

R2(config-if)#exit

R2(config)#ip route 0.0.0.0 0.0.0.0 200.0.0.1

R1#sh int tunnel 0

R1(config)#router eigrp 100

R1(config-router)#network 10.0.1.0 0.0.0.255

R1(config-router)#network 192.168.1.0 0.0.0.3

R2(config)#router eigrp 100

R2(config-router)#netwok

R2(config-router)#netwo

R2(config-router)#network 10.0.2.0 0.0.0.255

R2(config-router)#network 192.168.1.0 0.0.0.3

R2(config-router)#

%DUAL-5-NBRCHANGE: IP-EIGRP 100: Neighbor 192.168.1.1 (Tunnel0) is up: new adjacency

R2(config-router)#do sh ip route

GRE troubleshooting

Sh ip int br

Temp disable recursive routing

Sh int tunnel 0

Check source and destn address

Sh ip protocols

Change the ip to tunnel ip in protocol

Tunnel source is inside interface

Check the underlaying static route

BGP configuration :

IGP OSPF configuration:

Router0:

router ospf 1

log-adjacency-changes

network 0.0.0.0 255.255.255.255 area 0

Router 1:

router ospf 1

log-adjacency-changes

network 0.0.0.0 255.255.255.255 area 0

Router 2:

router ospf 1

log-adjacency-changes

passive-interface GigabitEthernet0/0

network 0.0.0.0 255.255.255.255 area 0

default-information originate

EGP:

router bgp 65000

bgp log-neighbor-changes

no synchronization

neighbor 100.0.0.1 remote-as 65001

network 1.1.1.1 mask 255.255.255.255

network 2.2.2.2 mask 255.255.255.255

network 3.3.3.3 mask 255.255.255.255

network 10.0.0.0 mask 255.255.0.0

network 100.0.0.0 mask 255.255.255.252

redistribute ospf 1 match external 1 external 2

ip route 0.0.0.0 0.0.0.0 100.0.0.1

ip route 10.0.0.0 255.255.0.0 Null0

Router 3:

router bgp 65001

bgp log-neighbor-changes

no synchronization

neighbor 100.0.0.2 remote-as 65000

neighbor 111.0.0.1 remote-as 65002

network 4.4.4.4 mask 255.255.255.255

network 111.0.0.0 mask 255.255.255.0

network 1.1.1.1 mask 255.255.255.255

network 2.2.2.2 mask 255.255.255.255

network 3.3.3.3 mask 255.255.255.255

network 100.0.0.0 mask 255.255.255.252

network 10.0.0.0 mask 255.255.0.0

network 112.0.0.0 mask 255.255.255.0

network 200.0.0.1 mask 255.255.255.255

Router 5:

router bgp 65002

bgp log-neighbor-changes

no synchronization

neighbor 111.0.0.2 remote-as 65001

neighbor 112.0.0.2 remote-as 65003

network 200.0.0.1 mask 255.255.255.255

network 100.0.0.0 mask 255.255.255.252

network 1.1.1.1 mask 255.255.255.255

network 2.2.2.2 mask 255.255.255.255

network 3.3.3.3 mask 255.255.255.255

network 10.0.0.0 mask 255.255.0.0

network 111.0.0.0 mask 255.255.255.0

network 112.0.0.0 mask 255.255.255.0

Internet:

router bgp 65003

bgp log-neighbor-changes

no synchronization

neighbor 112.0.0.1 remote-as 65002

network 130.0.2.0 mask 255.255.255.0

network 220.0.0.0

network 15.0.0.0

network 67.56.22.0 mask 255.255.255.0

network 112.0.0.0 mask 255.255.255.0

BGP troubleshooting:

Sh ip bgp summary

Sh ip bgp neighbor

Sh ip route

Sh ip bgp

HRSP:

Internet:

interface GigabitEthernet0/0/0

ip address 17.0.0.1 255.255.255.0

no shut

interface GigabitEthernet0/1/0

ip address 18.0.0.1 255.255.255.0

no shut

router bgp 65001

bgp log-neighbor-changes

no synchronization

neighbor 17.0.0.2 remote-as 65000

neighbor 18.0.0.2 remote-as 65000

network 15.0.0.0 mask 255.255.255.0

Router1:

interface GigabitEthernet0/0

ip address 12.0.0.1 255.255.255.252

interface GigabitEthernet0/1

ip address 10.10.10.2 255.255.255.0

interface GigabitEthernet0/2

ip address 10.20.20.3 255.255.255.0

interface GigabitEthernet0/0/0

ip address 17.0.0.2 255.255.255.0

router ospf 1

log-adjacency-changes

passive-interface GigabitEthernet0/0/0

network 0.0.0.0 255.255.255.255 area 0

router bgp 65000

bgp log-neighbor-changes

no synchronization

neighbor 17.0.0.1 remote-as 65001

network 17.0.0.0 mask 255.255.255.0

network 12.0.0.0 mask 255.255.255.252

network 10.10.10.0 mask 255.255.255.0

network 10.20.20.0 mask 255.255.255.0

ip route 0.0.0.0 0.0.0.0 17.0.0.1

standby version 2

standby 10 ip 10.10.10.1

standby 10 priority 110

standby 10 preempt

Router 2:

interface GigabitEthernet0/0

ip address 12.0.0.2 255.255.255.252

interface GigabitEthernet0/1

ip address 10.20.20.2 255.255.255.0

interface GigabitEthernet0/2

ip address 10.10.10.3 255.255.255.0

standby version 2

standby 10 ip 10.10.10.1

interface GigabitEthernet0/0/0

ip address 18.0.0.2 255.255.255.0

router ospf 1

log-adjacency-changes

passive-interface GigabitEthernet0/0/0

network 0.0.0.0 255.255.255.255 area 0

router bgp 65000

bgp log-neighbor-changes

no synchronization

neighbor 18.0.0.1 remote-as 65001

network 18.0.0.0 mask 255.255.255.0

network 12.0.0.0 mask 255.255.255.252

network 10.10.10.0 mask 255.255.255.0

network 10.20.20.0 mask 255.255.255.0

ip route 0.0.0.0 0.0.0.0 18.0.0.1

standby version 2

standby 10 ip 10.10.10.1

Switch 1:

interface FastEthernet0/1

switchport access vlan 10

switchport mode access

interface FastEthernet0/2

switchport access vlan 10

switchport mode access

Switch 2:

interface FastEthernet0/1

switchport access vlan 20

switchport mode access

interface FastEthernet0/2

switchport access vlan 20

switchport mode access

HRSP troubleshooting:

Duplicate address:

Sh standby <interface>

Sh standby g0/1

Check the standby version

Traceroute

Check preempt

ACL IPv4 and IPv6:

Deny host PC4 to access 10.4.4.100 server

R2(config)#access-list 1 deny host 10.2.2.12

R2(config)#access-list 1 permit any

R2(config-if)#ip access-group 1 out

Deny host PC5 to access 2001:DB8:22:22::/64

IPv6 only support named ACLs.

R1(config)#ipv6 access-list G0/2_IN

R1(config-ipv6-acl)#deny ipv6 host 2001:DB8:3:3::11 2001:DB8:22:22::/64

R1(config-ipv6-acl)#permit ipv6 any any

R1(config-ipv6-acl)#int g0/2

R1(config-if)#ipv6 traffic-filter G0/2_IN in

Deny PC3 communication with PC1:

R1(config)#ip access-list extended G0/1_IN

R1(config-ext-nacl)#deny ip host 10.2.2.11 host 10.1.1.11

R1(config-ext-nacl)#permit ip any any

R1(config-ext-nacl)#int g0/1

R1(config-if)#ip access-group G0/1_IN in

PC6 to telnet to R2:

R2(config)#ipv6 access-list TELNET

R2(config-ipv6-acl)#permit tcp host 2001:db8:3:3::12 any eq telnet

R2(config-ipv6-acl)#line vty 0 15

R2(config-line)#ipv6 access-class TELNET in

Troubleshooting:

Sh access-list <ACL no>

Check order of the statements

Sh ipv6 int g0/2

Sh run

SPAN:

Switch port analyzer:

Copies traffic from interface and send to another interface leads to monitoring server to analyze.

Switch 1 monitor G0/1 traffic

SW1(config)#monitor session 1 source interface g0/1 both

SW1(config)#monitor session 1 destination interface g0/2

IP SLA:

Service level agreement:

Ip sla 1

Icmp-echo 12.0.0.2

Icmp-echo <other side router>

Frequency 5

Exit

Ip-sla schedule 1 start-time now life forever

Ip-sla schedule 1 start-time <time> life <life time you want to send the ping>

Sh ip sla configuration

Sh ip sla statistics